ECA1803 – OPTOELECTRONICS AND OPTICAL COMMUNICATION

CO2 PROBLEM-SOLVING ASSIGNMENT

Submitted by: _______s.smrithi _______________________

Register Number: ____192412059_______________________

Department: __________ECE______________________

Course Outcome 2 | Problem Solving Assignment | Maximum Marks: 100

# 1. Stage 1 – Interpret the Communication Requirement:

The system to be designed is an eight-channel dense wavelength division multiplexing (DWDM) transmitter operating near 1550 nm. Each channel carries 25 Gb/s NRZ data, and the fiber link contains six amplified spans of 80 km each. The total fiber distance is therefore 480 km. The transmitter must keep each wavelength stable, provide enough optical power, support the data rate and limit nonlinear distortion.

## 1.1 Main design requirements:

| Parameter | Requirement | Design meaning |
| --- | --- | --- |
| Channel count and spacing | 8 channels; 100 GHz (about 0.8 nm) | Wavelengths must remain separated and stable. |
| Data rate | 25 Gb/s NRZ per channel | The source/modulator must support the required electrical bandwidth. |
| Fiber length | 6 × 80 km = 480 km | Dispersion and accumulated nonlinear effects matter. |
| Operating window | Around 1550 nm | Low fiber attenuation is available in this window. |
| Launch power | 2 mW per channel; 16 mW total | Power must be high enough for reception but controlled to limit nonlinearities. |
| Wavelength tolerance | ±0.1 nm | Temperature control and wavelength monitoring are required. |

## 1.2 Why source quality matters:

High modulation speed: the source and modulator must carry 25 Gb/s data without excessive pulse distortion.

Narrow linewidth and good side-mode suppression: unwanted spectral components can interfere with closely spaced DWDM channels.

Adequate output power: enough signal must reach the receiver after fiber and component losses, without excessive launch power.

Temperature stability: wavelength drift must be much smaller than the 0.8 nm channel spacing.

Low chirp and controlled dispersion: frequency changes during modulation can broaden pulses over a long fiber link.

## 1.3 Standards and requirement statement:

ITU-T G.957 provides optical-interface specifications for SDH equipment, while IEEE 802.3 optical PHY specifications address Ethernet interfaces. The exact interface version must be checked against the actual system rate and architecture; these documents should not be treated as interchangeable or as a complete DWDM channel-grid specification. ANSI Z136 is relevant to laser-product and accessible-emission safety. The final design must be verified against the applicable editions and product class.

Problem requirement statement: I need to design an eight-channel transmitter near 1550 nm that provides 25 Gb/s NRZ on each channel, maintains wavelength error within ±0.1 nm, supplies suitable launch power, and controls dispersion, nonlinear effects and accessible optical emission over a 480 km amplified link.

# 2. Stage 2 – Investigate and Select the Optical Source:

## 2.1 Emission wavelength and internal quantum efficiency:

The given bandgap energy is 0.80 eV. The approximate emission wavelength is calculated from the energy-wavelength relation.

λ (µm) ≈ 1.24 / E_g (eV)

Substitution: λ = 1.24 / 0.80 = 1.55 µm = 1550 nm.

The internal quantum efficiency is the fraction of recombination events that are radiative. Using the given radiative and non-radiative lifetimes:

η_int = τ_nr / (τ_r + τ_nr)

η_int = 40 ns / (10 ns + 40 ns) = 40/50 = 0.80 = 80%.

## 2.2 Comparison of candidate sources:

| Source | Strengths | Limitations for this link |
| --- | --- | --- |
| LED | Low cost; broad and simple source | Broad spectrum and limited modulation speed; unsuitable for tight DWDM spacing. |
| Fabry–Perot laser | Higher power and speed than an LED | Multiple longitudinal modes; weaker spectral purity. |
| DBR laser | Wavelength-selective grating and tunability | More complex than a basic laser; control is still needed. |
| DFB laser | Narrow spectrum, good single-mode operation and high speed | Needs wavelength/temperature management; suitable for DWDM. |
| VCSEL | Compact, efficient and easy to array | Typical wavelength and fiber-coupling choices may not suit a 1550 nm long-haul design without a specialized device. |
| Quantum-dot laser | Potentially low threshold and temperature-insensitive operation | Performance and availability depend on the specific device and maturity. |

## 2.3 Laser operation and selection:

Laser oscillation begins when the round-trip optical gain balances or exceeds the total cavity loss. Below threshold, spontaneous emission dominates; above threshold, stimulated emission produces a strong coherent output. A narrow linewidth and high side-mode suppression reduce spectral overlap and unwanted interference between adjacent DWDM channels.

I recommend a DFB laser for each DWDM channel. It offers single-mode emission, narrow linewidth, practical 25 Gb/s operation and suitable output power at 1550 nm. These characteristics fit the channel spacing and long transmission distance better than an LED or a multimode FP laser. A DBR laser is a credible alternative if broad wavelength tuning or reconfigurable channel assignment is a priority.

The selection remains conditional on checking the actual device datasheet for linewidth, side-mode suppression ratio, output power, modulation response and wavelength range. The calculated 1550 nm emission and 80% internal quantum efficiency are consistent with the target wavelength and the supplied lifetime data.

# 3. Stage 3 – Modulation, Chirp and Spectral Stability:

## 3.1 Output power at the operating current:

Given: threshold current I_th = 12 mA, slope efficiency η_s = 0.3 mW/mA and bias current I = 40 mA.

P = η_s (I − I_th)

P = 0.3 × (40 − 12) = 0.3 × 28 = 8.4 mW.

P(dBm) = 10 log₁₀[P(mW)] = 10 log₁₀(8.4) = 9.24 dBm

This is the laser output estimated from the supplied linear model. Actual power at the fiber input will be lower if coupling and modulator insertion losses are present.

## 3.2 NRZ bandwidth check:

B ≈ 0.7 R_b = 0.7 × 25 Gb/s = 17.5 GHz

The stated 3 dB modulation bandwidth is 20 GHz, so it is sufficient according to this approximate relation. The bandwidth margin is 20 − 17.5 = 2.5 GHz. This is a modest margin, so the complete transmitter response should be checked rather than relying on the laser bandwidth alone.

## 3.3 Rate equations and modulation:

A simplified carrier rate equation and photon rate equation can be written as:

dN/dt = I/(qV) − N/τ_n − G(N)S

dS/dt = ΓG(N)S + β_sp N/τ_sp − S/τ_p

Here N is carrier density, S is photon density, I is injection current, q is electron charge, V is active-region volume, G(N) is optical gain, Γ is the confinement factor and τ values are the relevant lifetimes. Relaxation oscillation is the coupled carrier-photon response that produces a resonance in the small-signal modulation response. Small-signal modulation studies response around a bias point; large-signal modulation includes the full switching between logic levels and can produce stronger transient chirp and overshoot.

## 3.4 Direct versus external modulation:

| Approach | Advantage | Concern |
| --- | --- | --- |
| Direct modulation | Simple, compact and low cost | Current changes can cause frequency chirp; chirp interacts with fiber dispersion. |
| External modulation | Laser can run at steady bias; lower chirp is possible with a suitable modulator | Adds insertion loss, cost and drive electronics. |

## 3.5 Wavelength drift and control:

Uncontrolled drift = 0.1 nm/°C × 40°C = 4 nm

A 4 nm shift is five times the approximate 0.8 nm channel spacing, so passive temperature stability is not acceptable. To keep the wavelength within ±0.1 nm, the temperature variation corresponding to that limit is ±0.1/0.1 = ±1°C. A thermoelectric cooler (TEC) with wavelength monitoring is recommended. Injection-current tuning is useful for fine adjustment, but current also changes output power and can introduce chirp; it should not be the only stability mechanism.

Design decision 2: I recommend external modulation with TEC-based wavelength control because it reduces chirp-related dispersion penalties and provides the temperature stability required by the DWDM grid.

4. Stage 4 – Investigate and Control Optical Nonlinearities:

## 4.1 Effective length and self-phase modulation:

The fiber attenuation is given as 0.2 dB/km. The effective-length equation requires the power attenuation coefficient in nepers per kilometre.

α = 0.2 / 4.343 = 0.04605 km⁻¹

L_eff = [1 − e^(−αL)] / α

For L = 80 km: L_eff = [1 − e^(−0.04605×80)] / 0.04605 ≈ 21.27 km.

For γ = 1.3 W⁻¹km⁻¹ and P = 2 mW = 0.002 W:

φ_NL = γ P L_eff = 1.3 × 0.002 × 21.27 ≈ 0.0553 rad per span

If each amplifier restores the launch power at the start of each span, the approximate accumulated phase over six spans is 6 × 0.0553 = 0.332 rad. This is a simplified estimate; the exact value depends on amplifier placement, channel loading and power evolution.

## 4.2 XPM and FWM:

Self-phase modulation (SPM) changes a channel’s own phase because its intensity changes the fiber refractive index.

Cross-phase modulation (XPM) occurs when power variations in neighbouring channels change the phase of the channel being observed. It increases with channel interaction and launch power.

Four-wave mixing (FWM) combines frequencies from multiple channels and can create new frequency components. Dispersion and the 100 GHz spacing affect phase matching; standard single-mode fiber dispersion can reduce, but does not guarantee elimination of, FWM.

## 4.3 SBS and SRS threshold estimates:

The given effective area is 80 µm² = 80 × 10⁻¹² m². For the threshold relations, L_eff must be in metres: 21.27 km = 21,270 m.

P_th(SBS) ≈ 21 A_eff / (g_B L_eff)

P_th(SBS) = 21(80×10⁻¹²) / [(5×10⁻¹¹)(21,270)] ≈ 0.00158 W = 1.58 mW.

P_th(SRS) ≈ 16 A_eff / (g_R L_eff)

P_th(SRS) = 16(80×10⁻¹²) / [(6×10⁻¹⁴)(21,270)] ≈ 1.00 W.

The 2 mW per-channel launch power is slightly above the simple SBS threshold estimate, so SBS deserves attention. It is far below the estimated SRS threshold. The total launch power is 8 × 2 = 16 mW. Threshold comparisons are approximate: SBS depends strongly on linewidth, polarization and the actual span conditions, while SRS can transfer power between channels.

## 4.4 Control and eye safety:

SBS can be reduced by using a source with an appropriate linewidth, avoiding unnecessary launch power and managing the optical spectrum. SPM/XPM/FWM penalties can be reduced through power optimization, suitable channel planning and dispersion-aware system design. The goal is not simply to lower power, because the receiver still needs adequate signal-to-noise ratio.

For a preliminary check, assume an accessible-emission screening limit of 10 mW. The 16 mW aggregate launch power exceeds this assumption, so enclosure, interlocking and a formal laser-safety assessment are required. This is only a project assumption, not a universal ANSI Z136 limit; the actual limit depends on wavelength, exposure time, beam geometry and product class.

Design decision 3: I recommend optimized channel power, linewidth control, dispersion-aware channel planning and an enclosed, interlocked optical path to limit nonlinear penalties while preserving received power.

5. Stage 5 – Soliton Transmission, Digital Backpropagation and Integration:

## 5.1 Soliton calculations:

A soliton can maintain its pulse shape when anomalous chromatic dispersion balances Kerr nonlinearity. The supplied pulse width is T₀ = 15 ps and |β₂| = 20 ps²/km.

L_D = T₀² / |β₂| = (15 ps)² / (20 ps²/km) = 225/20 = 11.25 km

At the given per-channel launch power P₀ = 2 mW = 0.002 W:

L_NL = 1 / (γP₀) = 1 / (1.3 × 0.002) = 384.6 km

N² = L_D / L_NL = 11.25 / 384.6 = 0.02925

N = √0.02925 ≈ 0.171

For a fundamental soliton, N = 1 and L_D = L_NL. Therefore, the required peak power from the supplied model is:

P₀(fundamental) = |β₂| / (γT₀²) = 20 / (1.3 × 225) ≈ 0.0684 W = 68.4 mW

The stated 2 mW launch power is much lower than the calculated 68.4 mW fundamental-soliton peak power, and N is far below 1. Soliton transmission is therefore not feasible under the given assumptions and launch power. Increasing power to the calculated value would need a new nonlinear, safety and link-budget assessment.

## 5.2 Digital backpropagation (DBP):

DBP numerically reverses an estimated fiber-propagation model to compensate chromatic dispersion and Kerr nonlinear effects. It can improve performance when the receiver has access to the complex optical field and the model, sampling rate and processing are sufficiently accurate. Its practical limitations include computational complexity, model mismatch and difficulty recovering the full field in a simple direct-detection receiver. For this 25 Gb/s NRZ link, full-link DBP should not be assumed as the default. Use it only if the selected receiver architecture supports the required coherent sampling and processing; otherwise use appropriate dispersion management and simpler DSP.

## 5.3 Integration alternatives:

| Implementation | Benefits | Trade-offs |
| --- | --- | --- |
| Discrete components | Flexible component selection and serviceability | Larger footprint, more optical connections and alignment losses. |
| Monolithic InP | Can integrate lasers, amplifiers and modulators compactly | Process and integration complexity; design flexibility depends on platform. |
| Si-photonics with hybrid laser | Scalable passive routing and WDM integration; suitable for multi-channel layouts | Hybrid laser coupling, packaging and thermal control remain important. |

For an eight-channel transmitter, silicon photonics with hybrid-integrated DFB lasers is a practical target because it supports compact WDM routing and scalability. Monolithic InP is a credible alternative where tight active-device integration and a qualified manufacturing platform are more important than silicon-photonic routing flexibility.

6. Stage 6 – Verification and Critical Evaluation:

The following table checks the calculations and choices against the original system requirement. “Satisfactory with conditions” means that the design is plausible but must still be verified with component datasheets and system-level measurements.

| Requirement | Calculated / selected result | Assessment |
| --- | --- | --- |
| Optical source | 1550 nm DFB laser per channel; narrow-linewidth, single-mode operation | Satisfactory, subject to datasheet verification |
| Output power | 8.4 mW = 9.24 dBm from the supplied slope-efficiency model | Potentially sufficient; include coupling/modulator losses |
| Modulation bandwidth | 17.5 GHz required; 20 GHz available; 2.5 GHz margin | Satisfactory but margin is modest |
| Wavelength stability | Uncontrolled drift 4 nm; target ±0.1 nm; TEC stability equivalent to ±1°C | Requires active temperature control and wavelength monitoring |
| Nonlinear effects | SPM ≈ 0.055 rad/span; SBS estimate ≈ 1.58 mW | Power and linewidth must be managed |
| SRS threshold | Approx. 1.00 W using supplied relation | 2 mW/channel is well below estimate |
| Soliton condition | L_D = 11.25 km; L_NL = 384.6 km; N ≈ 0.171 | Soliton operation not supported at 2 mW |
| DSP / compensation | DBP conditional on receiver architecture; not assumed by default | Use only if coherent field access and processing are available |
| Safety | 16 mW aggregate; exceeds illustrative 10 mW project assumption | Enclosure and formal safety classification required |
| Integration | Silicon photonics with hybrid-integrated lasers | Good scalability; packaging and thermal tests required |

## 6.1 Standards verification:

The transmitter must be checked against the exact optical interface selected for the final network. ITU-T G.957 is relevant to SDH optical interfaces, and the cited IEEE 802.3bs/bm documents cover specific Ethernet PHY generations rather than automatically defining every 25 Gb/s DWDM channel. The design review should verify wavelength range, transmitter power, spectral width, extinction ratio, eye mask, jitter, receiver compatibility and safety classification against the applicable interface and product standards. If the link is a proprietary DWDM system, the channel-grid and optical power budget also need their own requirements.

## 6.2 Overall consistency:

The calculations support a DFB-based transmitter with external modulation and active wavelength control. The bandwidth is adequate by the given approximation, but the margin is not large. Temperature control is essential because the uncontrolled wavelength drift is much larger than the channel spacing. The simple SBS estimate is close to the proposed per-channel launch power, so power and linewidth require attention. The soliton calculation does not support soliton operation at 2 mW. DBP should be selected only after the receiver architecture and processing budget are fixed.

7. Final Transmitter Design – Integrated Engineering Solution:

## 7.1 Proposed configuration:

The proposed design uses eight wavelength-controlled DFB laser channels near 1550 nm. Each laser is held at a stable bias and drives an external modulator. A TEC and wavelength monitor stabilize each channel, and a multiplexer combines the channels before the fiber link. Optical power is set after considering modulator and coupling losses. The transmitter and line interface are enclosed and interlocked where required by the final laser safety classification.

## 7.2 Final parameter table:

| Parameter | Adopted value / approach | Reason |
| --- | --- | --- |
| Number of channels | 8 | Matches the DWDM requirement |
| Channel spacing | 100 GHz (about 0.8 nm) | Keeps the channels separated |
| Nominal wavelength | 1550 nm | From E_g = 0.80 eV |
| Optical source | DFB laser per channel | Narrow-linewidth single-mode source |
| Bias current | 40 mA (given) | Above 12 mA threshold |
| Estimated laser output | 8.4 mW; 9.24 dBm | Calculated from slope efficiency |
| Data rate | 25 Gb/s NRZ per channel | Required line rate |
| Bandwidth | 17.5 GHz approximate requirement; 20 GHz available | 2.5 GHz estimated margin |
| Modulation | External modulation | Lower chirp potential over long fiber |
| Wavelength control | TEC plus wavelength monitoring; ±1°C equivalent temperature tolerance | Targets ±0.1 nm stability |
| Launch power | 2 mW/channel as the link-design starting point | Recheck SBS and receiver budget |
| Nonlinear strategy | Power optimization, linewidth control, channel planning and dispersion-aware design | Limits SPM/XPM/FWM and SBS risk |
| Soliton / DBP | No soliton at given power; DBP conditional on coherent receiver | Matches calculated feasibility |
| Integration | Silicon photonics with hybrid-integrated DFB lasers | Compact, scalable eight-channel routing |

## 7.3 Clearly labelled block diagram:

| DFB Laser 1–8 | TEC + wavelength monitor | External modulators | 8-channel DWDM MUX | Optical line output |
| --- | --- | --- | --- | --- |
| 1550 nm channels | Temperature feedback | 25 Gb/s NRZ data | 100 GHz spacing | To 6 × 80 km spans |
| Narrow linewidth | Wavelength lock | Low-chirp modulation | Combined optical signal | Enclosed / interlocked |

Control path: wavelength monitor → controller → TEC / laser temperature. The final receiver and amplifier design must confirm the optical power budget and signal quality over all six spans.

## 7.4 Final engineering decision:

YES, I would proceed with this transmitter as a design candidate, but not deploy it before verification. The DFB source, external modulation, TEC control and integrated WDM routing address the main system needs. Deployment depends on confirming the real component bandwidth and output power, the applicable interface standards, the optical safety classification, and the measured nonlinear performance of the full link.

8. Engineering Discussion and Final Conclusion:

## 8.1 Key decisions and their justification:

Source: DFB laser is selected because the 1550 nm DWDM link needs narrow spectral width, single-mode operation and high-speed capability.

Modulation: external modulation is preferred to reduce chirp-related penalties over a long multi-span link.

Wavelength stability: TEC feedback is necessary because a 40°C ambient change would otherwise cause about 4 nm of drift, compared with a tolerance of ±0.1 nm.

Nonlinear control: the estimated SPM phase is about 0.055 rad per span, while the simple SBS threshold is approximately 1.58 mW. The 2 mW/channel design therefore needs a more detailed SBS check.

Soliton transmission: the calculated order N ≈ 0.171 is below one, so the stated launch power does not meet the fundamental-soliton condition.

Compensation and integration: DBP is conditional on receiver capability. Silicon photonics with hybrid lasers is selected for compact routing and scalability, with thermal and packaging validation still required.

## 8.2 Limitations and checks before deployment:

The calculations use the simplified relations supplied in the assignment. The actual result can change with the laser linewidth, pulse shape, polarization, dispersion map, amplifier noise figure, connector loss, modulator insertion loss and channel loading. In particular, the given SBS/SRS threshold expressions provide first estimates and are not a replacement for a full link simulation or laboratory measurement.

The 10 mW eye-safety value used in Stage 4 is only an assumed project screening value. A formal assessment must use the relevant ANSI Z136 requirements, the product classification and the accessible emission at the point a person could reach. Similarly, the standards review must identify the exact optical interface rather than applying a standard solely because its name appears in the assignment.

## 8.3 Conclusion:

This study develops a practical starting design for an eight-channel, 25 Gb/s-per-channel DWDM transmitter near 1550 nm. The calculated emission wavelength is 1550 nm, internal quantum efficiency is 80%, estimated laser output is 8.4 mW, and the approximate bandwidth requirement is 17.5 GHz. The 20 GHz bandwidth meets the estimate with a 2.5 GHz margin. Active wavelength control is essential, and external modulation is preferred for better chirp management. Nonlinear effects—especially SBS near the assumed launch power—need careful validation. The supplied soliton parameters do not support soliton transmission at 2 mW, and DBP should be used only if the receiver can support it. A hybrid-laser silicon-photonics implementation offers a scalable direction, subject to standards, safety and end-to-end link verification.

## References :

1. Silicon photonics for high-speed communication (2024)

Zhou, X., Yi, D., Chan, D. W. U., et al. (2024). “Silicon photonics for high-speed communications and photonic signal processing.” npj Nanophotonics, 1, Article 27.

https://doi.org/10.1038/s44310-024-00024-7

Relevant to: high-speed optical modulators, integrated optical transmitters and multichannel communication.

2. Road mapping the next generation of silicon photonics (2024)

Shekhar, S., Bogaerts, W., Chrostowski, L., et al. (2024). “Roadmapping the next generation of silicon photonics.” Nature Communications, 15, Article 751.

https://doi.org/10.1038/s41467-024-44750-0

Relevant to: photonic integration, scalable optical communication devices and future transmitter design. Nature Communications

3. Optical solitons in multimode fibers : Recent advances (2024)

Zitelli, M. (2024). “Optical solitons in multimode fibers : recent advances.” Journal of the Optical Society of America B, 41(8), 1655–1664.

https://doi.org/10.1364/JOSAB.528242

Relevant to: soliton propagation, dispersion, nonlinear optical effects and long-distance transmission research.

4. Review on optical solitons for long-haul transmission (2024)

Kaur, P., Dhawan, D., & Gupta, N. (2024). “Review on optical solitons for long haul transmission.” Journal of Optical Communications, 45(s1), s1817–s1828.

https://doi.org/10.1515/joc-2023-0093

Relevant to: soliton-based optical transmission and nonlinear limitations in fiber communication.

5. 3D integrated silicon photonics transmitters for 224 Gbaud optical interconnects (2025)

“3D integrated silicon photonics transmitters for 224 Gbaud optical interconnects.” (2025). Optics Communications, 577, Article 131410.

https://doi.org/10.1016/j.optcom.2024.131410

Relevant to: integrated transmitter architecture, high-speed modulation and silicon-photonics implementation.

6. Integrating silicon photonics with complementary metal–oxide–semiconductor technologies (2025; published online)

Wan, Y., He, W., Jaussi, J., et al. “Integrating silicon photonics with complementary metal–oxide–semiconductor technologies.” Nature Reviews Electrical Engineering (published online, 2025; volume 3, pages 15–31, 2026).

Read the article