ECA1803 – OPTOELECTRONICS AND OPTICAL COMMUNICATION

CO3 SCENARIO-BASED ASSIGNMENT

Design and Performance Evaluation of an Integrated Photodetection, Imaging, Biomedical Sensing and Display System

Submitted by: _____________S.SMRITHI_____________________

Register Number: _____192412059_________________________

Department: _____ECE______________________________

Course Outcome 3 | Scenario-Based Assignment | Maximum Marks: 100

# Stage 1 – Interpret the Application Requirements:

The proposed platform combines wearable biomedical sensing, automotive LiDAR, imaging, UV detection, face recognition, auxiliary photovoltaic power and an advanced display. PPG must measure small changes in reflected light. LiDAR needs fast response and sensitivity to weak returning pulses. Imaging requires good resolution and low noise, while the display and power system must meet size, heat and energy limits.

## 1.1 Main requirements:

| Parameter | Given condition | Design implication |
| --- | --- | --- |
| PPG | R = 0.60 A/W; ΔP = 25 μW | Measure small optical changes. |
| LiDAR | P = 3 μW; R = 0.80 A/W | Detect weak returning light. |
| Low-light | Signal −22 dBm; noise −37 dBm | Calculate SNR and reduce noise. |
| Detector speed | Transit time = 20 ps | Estimate bandwidth. |
| PV | Voc = 0.60 V; Isc = 5 A; FF = 0.75 | Estimate maximum power. |
| Imaging/display | CMOS/CCD; OLED/MicroLED/QD | Select for performance and integration. |
| Simulation | MATLAB/SCILAB | Verify calculations and plot results. |

## 1.2 Required characteristics:

High responsivity and quantum efficiency to convert photons into photocurrent.

Low dark current and low noise for weak-signal detection.

Adequate bandwidth and short response time for LiDAR and fast imaging.

Good linearity so output current follows optical power without saturation.

Compact pixels, efficient readout and SoC integration for cameras and face recognition.

Suitable display brightness, lifetime, power consumption and thermal performance.

## Application requirement statement:

The platform must measure small optical changes in wearable sensors, detect weak and fast LiDAR returns, produce reliable images, support UV and NIR sensing, provide suitable display output and use power efficiently. Detector choices must be justified using photocurrent, SNR, bandwidth, noise and integration analysis.

# Stage 2 – Select the Photodetector Configuration:

## 2.1 PPG photocurrent:

Responsivity relates incident optical power to photocurrent. Given R = 0.60 A/W and ΔP = 25 μW:

ΔI = R × ΔP

ΔI = 0.60 × 25 × 10⁻⁶ = 15 × 10⁻⁶ A = 15 μA

The expected PPG current variation is 15 μA. PPG measures changes in light absorbed and reflected by tissue as blood volume changes with each heartbeat. The signal can be processed to estimate pulse rate and related trends.

## 2.2 LiDAR photocurrent:

I = RP = 0.80 × 3 × 10⁻⁶ = 2.4 μA

This is the ideal photocurrent before optical losses, dark current, receiver noise or avalanche multiplication.

## 2.3 PIN and APD comparison:

| Feature | PIN photodiode | APD |
| --- | --- | --- |
| Internal gain | No significant multiplication | Internal multiplication, M > 1 |
| Sensitivity | Suitable when light is adequate | Can improve weak-signal sensitivity |
| Noise | Lower multiplication noise | Avalanche excess noise added |
| Bandwidth | Often high | Gain and capacitance may limit speed |
| Bias | Usually lower | Higher bias; careful control needed |
| Complexity | Simple | More bias and temperature control |
| Suitable use | PPG and general imaging | LiDAR/low-light when gain is needed |

## 2.4 Detector recommendation:

I recommend a low-noise PIN photodiode for wearable PPG and ordinary imaging because it is simple and can measure the calculated 15 μA change. For LiDAR and very weak returns, an APD is a suitable option if additional sensitivity is needed. Final selection depends on wavelength, range, ambient light, required bandwidth and measured noise. Dark current must be minimized because it adds noise even without useful illumination.

Quantum efficiency is the fraction of incident photons that generate collected charge. NEP is the optical power that produces a signal equal to noise in a 1 Hz bandwidth; lower NEP indicates better sensitivity. Detectivity D* normalizes sensitivity for detector area and bandwidth. NEP and detectivity cannot be calculated numerically from the supplied data because noise spectral density and detector area are not given.

# Stage 3 – Noise and Signal-to-Noise Ratio:

## 3.1 SNR calculation

Given signal power = −22 dBm and noise power = −37 dBm. When both are in dBm, their difference gives SNR in dB.

SNR(dB) = P_signal(dBm) − P_noise(dBm)

SNR = (−22) − (−37) = 15 dB

SNR(linear) = 10^(15/10) ≈ 31.62

The signal power is about 31.6 times the noise power at this operating point.

## 3.2 Main noise sources:

Shot noise is caused by the random arrival of photons and discrete electric charge; it increases with current and bandwidth.

Dark-current noise comes from current generated inside the detector without useful light; it matters for weak signals.

Avalanche noise arises from the statistical nature of APD carrier multiplication.

Thermal noise is caused by random motion of charge in receiver resistors and electronics.

## 3.3 SNR improvement strategy:

| Method | Benefit | Design caution |
| --- | --- | --- |
| Suitable APD gain | Raises weak signal current | Excess noise and bias must be considered. |
| Optical filtering | Rejects unwanted ambient light | Filter must match source wavelength. |
| Reduce dark current | Improves weak-light performance | Select suitable detector; cooling may help. |
| Optimize bandwidth | Reduces integrated noise | Must still pass the wanted signal. |
| Improve optical coupling | More wanted light reaches detector | Alignment and package design matter. |
| Shield/stabilize receiver | Reduces stray light and drift | Adds packaging requirements. |

## Design decision 2:

I recommend a low-noise receiver with optical filtering, optimized bandwidth and only the APD gain required for the application. The calculated SNR is 15 dB. The dominant noise source cannot be identified from the two power values alone; detector dark current, bandwidth and noise measurements are needed. If received power falls while dark current remains constant, SNR becomes worse.

# Stage 4 – Response Time and High-Speed Detection:

## 4.1 Transit-time bandwidth:

The given transit time is tr = 20 ps and the assignment uses f ≈ 0.44/tr.

tr = 20 ps = 20 × 10⁻¹² s

f ≈ 0.44 / (20 × 10⁻¹²) = 22 × 10⁹ Hz = 22 GHz

The approximate transit-time-limited bandwidth is 22 GHz. The complete detector and receiver bandwidth may be lower.

## 4.2 Speed limitations:

Transit time: carriers need time to cross the active region. A thicker region can absorb more light but may slow the response.

RC time constant: detector capacitance and load resistance create τ = RC.

Detector capacitance: a larger active area may improve light collection but increase capacitance and reduce bandwidth.

Receiver electronics, package parasitics and interconnects can limit system bandwidth.

Approximate RC bandwidth: f_RC ≈ 1 / (2πRC)

## 4.3 High-speed configuration and decision:

I recommend a small-area, low-capacitance PIN detector for high-speed imaging when signal levels are sufficient, and an APD for LiDAR when its sensitivity advantage is needed and its bandwidth remains adequate. The 22 GHz estimate supports fast detection, but final selection must use the datasheet and full receiver response. The design must balance active area, light collection, sensitivity and speed.

# Stage 5 – PV, Image Sensors and Emerging Displays:

## 5.1 Photovoltaic analysis

Given Voc = 0.60 V, Isc = 5 A and fill factor FF = 0.75:

Pmax = Voc × Isc × FF

Pmax = 0.60 × 5 × 0.75 = 2.25 W

The estimated maximum power is 2.25 W under the stated conditions. Voc is voltage at zero output current; Isc is current at zero terminal voltage. Fill factor indicates how close the I–V curve comes to an ideal rectangle. Efficiency is η = Pmax/Pin, so incident optical power is also needed to calculate efficiency.

Flexible PV can conform to wearable surfaces and provide auxiliary energy, but efficiency, bending durability, moisture protection and temperature behaviour must be checked.

## 5.2 CMOS versus CCD:

| Feature | CMOS | CCD |
| --- | --- | --- |
| Readout | Local pixel circuitry; often parallel | Charge shifted through array to readout node(s) |
| Speed/power | Usually fast and lower power | Can be uniform but often higher power |
| Integration | Strong SoC compatibility | May require more external support |
| Noise | Modern designs can have low noise | Traditionally valued for uniformity/low read noise |
| Applications | Face recognition, automotive, compact cameras | Specialized scientific/low-light uses |

Smaller pixels increase resolution for a given sensor area, but collect fewer photons and may reduce full-well capacity, low-light sensitivity and dynamic range. Pixel design, microlenses and processing must be considered together.

## 5.3 Display technologies:

| Technology | Strengths | Limitations / use |
| --- | --- | --- |
| OLED | Self-emissive, deep blacks, high contrast, flexible | Burn-in/lifetime considerations; suitable for foldable displays. |
| MicroLED | High brightness, fast response, potential long lifetime | Manufacturing and mass-transfer cost; promising for AR/MR. |
| Quantum dot | Strong colour gamut and colour conversion | Performance depends on architecture; QD-LCD may need backlight. |

Design decision 4: I recommend flexible PV for auxiliary wearable power when adequate, CMOS for compact imaging and face recognition, and OLED for a foldable consumer display. MicroLED is an alternative for high-brightness AR/MR if manufacturing cost can be met.

# Stage 6 – Biomedical and Industrial Applications:

## 6.1 Wearable PPG:

An LED illuminates the skin and a photodiode measures light transmitted through or reflected from tissue. Blood-volume changes cause periodic intensity changes. The expected current change is 15 μA. A practical system uses suitable red, green or infrared illumination, a low-noise photodiode, ambient-light rejection, an analog front end and digital filtering. Low power and stable skin contact are important.

## 6.2 NIR spectroscopy:

Near-infrared light interacts with biological materials through wavelength-dependent absorption and scattering. The detector measures returned or transmitted light, and processing compares selected wavelengths. Source wavelength, detector responsivity, calibration, filtering and noise control determine quality. The brief does not specify target wavelengths or analytes, so a specific medical quantity cannot be calculated.

## 6.3 Automotive LiDAR:

LiDAR sends short optical pulses and detects their reflections. Range is estimated from round-trip time:

Range = cΔt / 2

The given 3 μW return and responsivity of 0.80 A/W produce 2.4 μA. An APD can improve sensitivity for weak returns, but ambient sunlight, eye safety, bandwidth, temperature and false detections must be managed.

## 6.4 Face recognition and vehicle sensing:

A CMOS image sensor is practical for face recognition because it offers compact integration, fast readout and low-power operation. Image quality depends on resolution, pixel size, illumination, exposure and processing. NIR illumination can help in low visible light if the source and detector are matched and eye-safety requirements are met.

## 6.5 UV sensing and application decision:

UV sensing requires a detector whose spectral response matches the target UV band and rejects unwanted visible and infrared light. UV-enhanced silicon or specialized wide-bandgap detectors may be suitable depending on wavelength and environment. Filters, calibration, packaging and temperature stability are important.

I recommend a PIN-based PPG module, an APD-based LiDAR receiver when weak-return sensitivity is needed, CMOS for face and automotive imaging, a wavelength-matched UV detector, and filtered NIR photodetection for spectroscopy. Each module should have a suitable filter, analog front end and software processing.

# Stage 7 – MATLAB/SCILAB Simulation and Verification:

The MATLAB script below plots photocurrent against optical power, calculates SNR and estimates detector bandwidth. The illustrative SNR plot assumes fixed noise. Detailed shot, dark-current and APD excess-noise simulation needs additional device parameters.

## 7.1 MATLAB code:

clc; clear; close all;

R = 0.60;                 % Responsivity (A/W)

P_uW = 0:5:100;           % Optical power (microW)

I_uA = R * P_uW;          % Current (microA)

figure;

plot(P_uW, I_uA, 'LineWidth', 2);

grid on;

xlabel('Optical Power (microW)');

ylabel('Photocurrent (microA)');

title('Photocurrent vs Optical Power');

I_PPG_uA = 0.60 * 25;

I_LiDAR_uA = 0.80 * 3;

Ps_dBm = -22; Pn_dBm = -37;

SNR_dB = Ps_dBm - Pn_dBm;

SNR_linear = 10^(SNR_dB/10);

tr = 20e-12;

f_GHz = (0.44/tr)/1e9;

fprintf('PPG current change = %.2f microA\n', I_PPG_uA);

fprintf('LiDAR current = %.2f microA\n', I_LiDAR_uA);

fprintf('SNR = %.2f dB (linear %.2f)\n', SNR_dB, SNR_linear);

fprintf('Bandwidth = %.2f GHz\n', f_GHz);

signal_dBm = -40:2:-10;

snr_curve = signal_dBm - Pn_dBm;

figure;

plot(signal_dBm, snr_curve, 'LineWidth', 2);

grid on;

xlabel('Signal Power (dBm)');

ylabel('SNR (dB)');

title('SNR vs Signal Power (Fixed Noise)');

## 7.2 Expected results:

| Quantity | Theoretical result | Interpretation |
| --- | --- | --- |
| PPG current change | 15 μA | Linear responsivity model. |
| LiDAR photocurrent | 2.4 μA | Before losses and noise. |
| SNR | 15 dB; ratio ≈ 31.62 | Signal power/noise power. |
| Bandwidth | 22 GHz | Transit-time estimate only. |
| PV maximum power | 2.25 W | From Voc, Isc and FF. |

The first plot is linear because responsivity is treated as constant. In the second plot, SNR increases one decibel for each one-decibel increase in signal power when noise is fixed. Run the code in MATLAB and save both plots for submission.

# Final System Design, Verification and Conclusion:

## 8.1 Integrated platform:

| Module | Selected configuration | Result / reason |
| --- | --- | --- |
| PPG | LED + PIN + filter/signal conditioning | 15 μA change. |
| LiDAR | Pulsed source + APD option + wideband amplifier | 2.4 μA ideal current. |
| Low-light | Optical filter, low-dark-current detector | SNR = 15 dB. |
| Fast detection | Low-capacitance detector and wideband receiver | 22 GHz transit estimate. |
| Auxiliary power | Flexible PV where practical | Pmax = 2.25 W. |
| Imaging | CMOS sensor + SoC | Compact, fast integration. |
| Display | OLED for foldable; MicroLED for bright AR/MR | Depends on cost and brightness. |
| NIR / UV | Wavelength-matched detector and filter | Choose after target wavelength is set. |
| Simulation | MATLAB/SCILAB models | Checks calculations and plots. |

## 8.2 Final verification:

| Requirement | Result | Status |
| --- | --- | --- |
| PPG photocurrent | 15 μA | Calculated |
| LiDAR photocurrent | 2.4 μA | Calculated |
| Low-light SNR | 15 dB | Calculated |
| Bandwidth | 22 GHz approximate | Transit-time estimate |
| PV maximum power | 2.25 W | Calculated |
| Quantum efficiency | Wavelength not given | More data needed |
| NEP/detectivity | Noise spectrum/area not given | More data needed |
| Sensor/display | CMOS; OLED; MicroLED alternative | Recommended |
| Simulation | MATLAB code supplied | Run and save plots |
| Safety/reliability | Product testing required | Not yet verified |

## 8.3 System block diagram:

| Input / source | Detection module | Processing | Output |
| --- | --- | --- | --- |
| LED / NIR / UV source | PIN/APD + optical filter | Analog front end + ADC | PPG, spectroscopy, UV result |
| Pulsed LiDAR source | Fast APD/PIN receiver | Timing and range processing | Object distance |
| Scene + lens | CMOS image sensor | ISP / SoC / recognition | Face/vehicle imaging |
| Flexible PV | Power management | Regulation and battery | Auxiliary wearable power |
| Processor / interface | Display driver | Image and brightness control | OLED / MicroLED / QD display |

## 8.4 Final engineering decision:

YES, I would continue with this platform as a design proposal, but I would not deploy it before hardware testing. The calculated PPG current, LiDAR current, SNR, bandwidth and PV power provide a useful starting point. The final design must be validated for detector noise, wavelength response, optical coupling, receiver bandwidth, display power, thermal behaviour and product safety. NEP, detectivity and quantum efficiency need additional device data.

## References :

1. Datasheets for the selected PIN/APD, CMOS sensor, display and PV modules; record manufacturer, model and revision.

2. MATLAB or GNU Octave documentation for plotting and numerical calculations.