ECA1803 – OPTOELECTRONICS AND OPTICAL COMMUNICATION

CO5 CASE STUDY BASED ASSIGNMENT

DWDM METRO LINK COMMISSIONING

From Field Measurements to a Standards-Based Upgrade Decision

Submitted by:  S. SMRITHI

Register Number:  192412059

Department:  Electronics and Communication Engineering

Course Outcome 5  |  Case Study Assignment  |  Maximum Marks: 100

# 1. Stage 1 – Interpret the Measurement / Performance Requirement:

The case concerns a 60 km ITU-T G.652.D fibre link carrying eight 10 Gb/s NRZ channels on a 100 GHz ITU-T G.694.1 grid. Channel 3 has an open eye, while Channel 5 has a partially closed eye and high BER. The received power margin is also below the required 3 dB. The system must be repaired and evaluated for a 1.6 Tb/s upgrade without laying new fibre.

| Instrument | What it validates | Use in this case |
| --- | --- | --- |
| Optical power meter | Absolute optical power and insertion loss | Check launch/received power and confirm repair loss. |
| Optical spectrum analyser (OSA) | Channel wavelengths, channel powers and OSNR | Verify grid, power equalization and noise. |
| OTDR | Event distance, event loss and reflectance | Locate the mechanical splice and connector faults. |
| Eye / BER tester | Eye opening, jitter, Q-factor and BER | Diagnose Channel 5 and confirm recovery. |
| Dispersion / PMD analyzer | Chromatic dispersion and polarization delay | Check whether fibre dispersion is within tolerance. |

Performance requirement statement: Repair the excessive-loss and reflective events, restore at least 3 dB receiver margin, and achieve BER ≤ 10⁻¹² (Q ≥ 7.03) while preserving the 60 km fibre. Then select an upgrade that supports 1.6 Tb/s with suitable grid spacing, OSNR margin and energy per bit.

# 2. Stage 2 – Link Budget, Dispersion and OSNR:

## 2.1 Fibre and as-built loss:

Fibre attenuation = 60 km × 0.21 dB/km = 12.60 dB.

Connector loss = 3 × 0.5 dB = 1.50 dB. Splice loss = 12 × 0.05 + 0.62 = 1.22 dB, treating Event 2 as the measured mechanical splice. Demultiplexer loss = 6.0 dB.

As-built total loss = 12.60 + 1.50 + 1.22 + 6.0 = 21.32 dB, using the listed measured connector/splice values. The assignment’s verification table gives 22.52 dB as the as-built value, which differs by 1.20 dB; this matches Event 3’s measured connector loss and should be reconciled against the power-meter measurement and whether the event is included in the connector total.

Design allowance = 4 × 0.5 + 13 × 0.05 = 2.65 dB for connectors and splices. Total design loss including fibre and demultiplexer = 12.60 + 2.65 + 6.0 = 21.25 dB.

The stated launch power is +1 dBm per channel. Using the 22.52 dB as-built figure, received power before penalties is +1 − 22.52 = −21.52 dBm. Using the component-sum figure of 21.32 dB gives −20.32 dBm. The field power-meter reading must resolve this inconsistency before acceptance.

## 2.2 Dispersion penalty:

Given δ = 5 log₁₀[1 + 2π(B·D·L·σλ)²], with B = 10 Gb/s, D = 17 ps/(nm·km), L = 60 km and σλ = 0.03 nm.

B·D·L·σλ = 10×10⁹ × 17×10⁻¹² × 60 × 0.03 = 0.306.

δ = 5 log₁₀[1 + 2π(0.306)²] ≈ 1.09 dB.

This is the dispersion penalty using the relation supplied in the assignment. The exact penalty depends on the definition of B and the source spectral-width assumptions.

## 2.3 Extinction-ratio penalty and receiver margin:

The brief gives the receiver sensitivity as −24 dBm at BER 10⁻¹² when ER ≥ 8.2 dB. Channel 3 has ER = 9.0 dB and satisfies this condition. Channel 5 has ER = 4.5 dB and does not; its sensitivity cannot be treated as −24 dBm without an additional ER penalty model.

For Channel 3, using the 22.52 dB loss and the 1.09 dB dispersion penalty, effective received level ≈ −21.52 − 1.09 = −22.61 dBm. Margin over −24 dBm ≈ 1.39 dB, below the required 3 dB.

For Channel 5, the same estimate before its unknown ER penalty is −22.61 dBm. Its actual margin is lower because ER = 4.5 dB is below the stated sensitivity condition. The assignment does not supply a formula for converting the ER shortfall into a precise dB penalty, so a numeric ER penalty should not be invented.

## 2.4 OSNR and PMD:

Using the supplied relation OSNR = 58 + P_in − NF, with P_in = −6 dBm per channel and NF = 5 dB: OSNR = 58 − 6 − 5 = 47 dB (0.1 nm). This is above the legacy link requirement and the listed coherent upgrade thresholds in a simplified calculation. Actual receiver OSNR must be measured at the receive end because the formula and stated amplifier input conditions do not fully describe every span and loss.

PMD estimate = 0.1 ps/√km × √60 ≈ 0.775 ps. The 10 Gb/s bit period is 100 ps; 10% is 10 ps. The estimated PMD is well below 10 ps.

Design Decision 1: I recommend repairing excess loss and restoring the required power margin before increasing launch power. Channel 3 is short of the 3 dB margin under the stated loss/penalty estimate, and Channel 5 also has a serious extinction-ratio and eye-quality issue. Confirm the total loss with a power meter because the brief’s 22.52 dB value and component sum do not agree.

# 3. Stage 3 – OTDR, Splice and Connector Configuration:

| Event | Distance | Measured event | Interpretation / action |
| --- | --- | --- | --- |
| 1 | 12.00 km | 0.05 dB, non-reflective | Low-loss fusion splice or benign event; document and monitor. |
| 2 | 31.50 km | 0.62 dB, reflectance −34 dB | High-loss mechanical splice; replace with a fusion splice if feasible. |
| 3 | 59.900 km | 1.20 dB, reflectance −22 dB | Poor/contaminated/damaged LC/UPC connector; highest repair priority. |
| 4 | 59.909 km | Reflective, −40 dB; loss unresolved | Second connector 9 m after Event 3; close spacing makes separate loss measurement difficult. |

## 3.1 Pulse width and dead-zone decision:

The events at 59.900 km and 59.909 km are only 9 m apart. The 10 ns setting has event dead zone 1 m and attenuation dead zone 4 m, so it is the best of the supplied settings for separating these events. The 100 ns and 1 μs settings have attenuation dead zones of 15 m and 100 m, respectively, and cannot reliably resolve a 9 m spacing for separate loss measurement.

Approximate spatial pulse length = c·τ/(2n), with n = 1.4682. For 10 ns: 1.02 m; for 100 ns: 10.21 m; for 1 μs: 102.1 m. The 10 ns pulse is therefore preferred near Events 3 and 4. Its dynamic range is 20 dB, so it may not be sufficient for the entire one-way trace loss plus the required 6 dB reserve. Use a longer pulse for the overall link trace and a short pulse for close event separation, with launch/receive fibre and suitable test direction.

Event round-trip delay is t = 2nL/c. Event 1 at 12.00 km: about 117.6 μs; Event 2 at 31.50 km: about 308.5 μs; Event 3 at 59.900 km: about 586.9 μs; Event 4 at 59.909 km: about 587.0 μs.

Location uncertainty from ±0.002 group-index error is approximately ΔL = L(Δn/n). At 59.9 km this is about ±81.6 m, much larger than the 9 m separation, so the stated group-index uncertainty must be calibrated/reduced before claiming precise separation based on distance alone.

## 3.2 Repair choice and verification:

First priority: clean, inspect and replace the Event 3 LC/UPC patch-panel connector; its 1.20 dB loss and −22 dB reflectance are the worst reported event.

Second priority: replace Event 2’s 0.62 dB mechanical splice with a low-loss fusion splice if access and repair policy permit.

Inspect Event 4’s mating connector and patch cord; use a compatible, clean connector interface. Do not mate UPC and APC connector end faces.

After repair, repeat bidirectional OTDR with short pulse for close events and longer pulse for end-to-end dynamic range. Confirm total insertion loss and received power using a calibrated power meter.

Design Decision 2: I recommend repairing Event 3 first, then Event 2, while separately inspecting Event 4. Use short-pulse OTDR near the end-panel events and a longer pulse for the full span; verify with both OTDR and end-to-end power measurements.

# 4. Stage 4 – Eye Diagram, BER and Quantum Limit:

## 4.1 BER from Q-factor:

Using BER = ½ erfc(Q/√2):

Channel 3, Q = 7.6: BER ≈ 1.5 × 10⁻¹⁴, below the 10⁻¹² target.

Channel 5, Q = 4.7: BER ≈ 1.3 × 10⁻⁶, above the 10⁻¹² target and therefore unacceptable.

## 4.2 Jitter and eye width:

At BER 10⁻¹², TJ = DJ + 14.07·RJ.

Channel 3: TJ = 0.18 + 14.07(0.012) = 0.34884 UI; estimated horizontal eye width = 1 − TJ = 0.65116 UI.

Channel 5: TJ = 0.30 + 14.07(0.022) = 0.60954 UI; estimated horizontal eye width = 0.39046 UI.

Channel 5 has larger deterministic and random jitter, so its timing margin is smaller. Vertical eye closure is associated with noise, amplitude distortion and the low ER; horizontal closure reflects timing uncertainty.

## 4.3 Is the Channel 5 problem shared or channel-specific?

Because Channel 3 on the same fibre has Q = 7.6 and ER = 9.0 dB while Channel 5 has Q = 4.7 and ER = 4.5 dB, the evidence points to a channel-specific transmitter or channel path problem in addition to the shared link-margin problem. Check Channel 5 transmitter bias/modulation, optical power, extinction ratio, connector cleanliness and receiver settings. Repair the shared fibre/connector loss as well, then retest both channels.

## 4.4 Quantum-limit calculation:

For ideal direct detection, BER = ½ exp(−N₁). For BER = 10⁻¹², N₁ = −ln(2×10⁻¹²) ≈ 26.94 photons per one-bit.

At 1550 nm, photon energy Eγ = hc/λ ≈ 1.282 × 10⁻¹⁹ J. At 10 Gb/s, the ideal received power corresponding to 26.94 photons/bit is P ≈ 26.94 × Eγ × 10¹⁰ ≈ 3.45 × 10⁻⁸ W = −44.62 dBm.

Compared with −24 dBm sensitivity, the ideal direct-detection limit is about 20.62 dB lower in power. This is an ideal quantum limit; real receivers require more power due to noise, losses and implementation penalties.

For the supplied PSK relations, homodyne BER = ½ erfc(√(2N)) and heterodyne BER = ½ erfc(√N). Numerical photon counts require solving these equations for N at BER 10⁻¹². These coherent limits cannot be compared fairly using only a direct-detection photon count without evaluating the respective erfc inverse.

Design Decision 3: Channel 5 is not acceptable. Correct the transmitter/channel-specific ER and jitter issue, repair common path losses, and verify Q ≥ 7.03, BER ≤ 10⁻¹², ER ≥ 8.2 dB and a sufficiently open eye on the BER tester.

# 5. Stage 5 – Integrated WDM Upgrade and Energy Strategy:

## 5.1 G.694.1 channel grid:

The grid anchor is 193.1 THz. For a 50 GHz grid, channel n = +3 is f = 193.1 THz + 3(0.05 THz) = 193.25 THz. Wavelength λ = c/f ≈ 299,792,458 / 193.25×10¹² ≈ 1551.0 nm.

Option Q has 32 GBd and roll-off α = 0.1, so approximate Nyquist occupied bandwidth = Rs(1+α) = 32 × 1.1 = 35.2 GHz. At 12.5 GHz slot granularity, slots required = ceil(35.2/12.5) = 3 slots, or 37.5 GHz allocated.

## 5.2 Upgrade comparison

| Option | Capacity | Spectrum / grid | OSNR required | Module power | Total module power | Energy per bit |
| --- | --- | --- | --- | --- | --- | --- |
| P: 16 × 100G DP-QPSK | 1.6 Tb/s | 16 × 50 GHz = 800 GHz | 12.5 dB | 22 W | 352 W | 220 pJ/bit |
| Q: 8 × 200G DP-16QAM | 1.6 Tb/s | 8 × 37.5 GHz minimum = 300 GHz (3 flex slots each) | 19.5 dB | 20 W | 160 W | 100 pJ/bit |
| R: 4 × 400G Si-photonic | 1.6 Tb/s | 4 × 75 GHz = 300 GHz | 26 dB | 15 W | 60 W | 37.5 pJ/bit |
| Legacy 10G SFP+ | 10 Gb/s per module | 8 existing channels = 80 Gb/s total | Not specified here | 3.5 W | 28 W for 8 modules | 350 pJ/bit per 10G module |

Energy per bit is estimated as module power divided by module payload rate. Option P: 22 W/100 Gb/s = 220 pJ/bit per module. Option Q: 20 W/200 Gb/s = 100 pJ/bit. Option R: 15 W/400 Gb/s = 37.5 pJ/bit. Legacy: 3.5 W/10 Gb/s = 350 pJ/bit.

Using the simplified OSNR calculation of 47 dB, OSNR margin relative to the option requirements is P: 34.5 dB; Q: 27.5 dB; R: 21 dB. These are preliminary margins only; receiver OSNR must be measured after the repaired link and amplifier configuration is finalized.

Option R has the lowest module power and energy per bit, but the highest OSNR requirement. Option Q uses the same nominal 300 GHz occupied allocation as R under the given assumptions and has lower OSNR requirement than R, while using more total module power. Option P has the widest spectrum and largest module power.

Design Decision 4: I recommend Option Q as a balanced initial upgrade candidate if the measured OSNR is adequate and the flex-grid equipment is supported. Option R is the energy-efficient alternative, but should only be selected if measured OSNR margin, vendor reach specifications and interoperability are verified. Do not commit to either option until the link is repaired and the receiver OSNR is measured.

# 6. Stage 6 – Verification and Critical Evaluation:

| Requirement | Target | Current analysis / acceptance evidence |
| --- | --- | --- |
| Total optical link loss | ≤ 21.25 dB design value | Component sum is 21.32 dB; brief also states 22.52 dB as-built. Reconcile by calibrated power-meter measurement after repairs. |
| Received optical power | At least −21 dBm for −24 dBm sensitivity plus 3 dB margin, before any additional penalties | Estimated −21.52 dBm before dispersion penalty using 22.52 dB loss; below target. |
| System margin | ≥ 3 dB | Not met by the current estimate; repair excess loss and remeasure. |
| OTDR event condition | No event above accepted loss/reflectance allowance | Event 3 (1.20 dB, −22 dB) and Event 2 (0.62 dB) require repair/inspection. |
| Eye / Q / jitter / ER | Q ≥ 7.03; ER ≥ 8.2 dB; BER ≤ 10⁻¹² | Channel 3 passes Q/ER; Channel 5 fails Q/ER and has reduced eye width. |
| Upgrade capacity | 1.6 Tb/s | P, Q and R each nominally provide 1.6 Tb/s. |
| OSNR | P ≥ 12.5 dB; Q ≥ 19.5 dB; R ≥ 26 dB | Simplified estimate 47 dB; confirm with OSA at receiver after repair. |
| Energy per bit | Minimize while meeting OSNR and reach | R lowest at 37.5 pJ/bit; Q 100 pJ/bit; P 220 pJ/bit, based on module power only. |

## 6.1 Final workflow:

Figure 1. Measurement, repair, verification and upgrade workflow.

## 6.2 Final engineering decision:

NO — I would not deploy the link in its current condition. The estimated receiver margin is below 3 dB, Channel 5 fails the BER and extinction-ratio requirements, and the measured-loss figures in the brief require reconciliation. I would clean/replace the poor end connector first, replace the high-loss mechanical splice, inspect the adjacent connector, and then repeat OTDR, insertion-loss, OSA/OSNR and eye/BER tests. After the link passes the ≥3 dB margin and BER ≤10⁻¹² acceptance criteria, Option Q is a balanced upgrade candidate; Option R can be considered if its higher OSNR requirement is demonstrated with margin and vendor reach is verified. This staged decision avoids deploying a high-capacity system on a link that has not passed commissioning criteria.

# References :

1. ITU-T G.652.D, characteristics of a single-mode optical fibre and cable.

2. ITU-T G.694.1, spectral grids for WDM applications: DWDM frequency grid.

3. OTDR, optical spectrum analyser, optical power meter and transceiver datasheets used for the final field verification; add manufacturer/model and revision when available.